PROJECT:	T1P1 Activity on Lawful Intercept for UMTS

TITLE:	Recommended changes to TS 33.108 to address Timing requirements related with Lawful Intercept Capabilities for GPRS Packet Domain.

SOURCE:	Lou Degni

CALEA Implementation Section

14800 Conference Center Drive, Suite 300

Chantilly, VA 20151-3810

Tel:  	(703) 814-4729

Fax:  	(703) 814-4720

Sandra Lopez

Telcordia Technologies, Inc.

331 Newman Springs Road

Red Bank, NJ 07701

Tel:	(732) 758-3222

Fax:  	(732) 758-4196

e-mail:	slopez@telcordia.com

DATE:	March 26-28, 2002

LOCATION: 	Irving, Texas

ABSTRACT:	This contribution proposes text to address Issue #5, Timing requirements, in TS 33.108 as documented in the issues list published in T1P1/2002-004 .

DISTRIBUTION:	T1P1.SAH

NOTICE

©2002 Telcordia Technologies, Inc. The proposals in this submission have been formulated to assist committee T1P1.  This document is offered to the committee as a basis for discussion and is not binding on Telcordia Technologies. The requirements are subject to change in form and numerical value after more study. Telcordia Technologies specifically reserves the right to add to, or amend, the quantitative statements made herein.  Nothing contained herein shall be construed as conferring by implication, estoppel, or otherwise any license or right under any patent, whether or not the use of information herein necessarily employs an invention of any existing or later issued patent.

[PAGE INTENTIONALLY LEFT BLANK]

A. Introduction

This contribution proposes text to address shortcomings in TS 33.108 in the area of timing requirements. This contribution addresses Issue # 5 of the issues list documented in T1P1/2002-004 and action items assigned to CIS/Telcordia during the T1P1.SAH conference calls held on January 8, 2002 and January 14, 2002.

This contribution builds on contribution T1P1/2001-112 R0 which was submitted for discussion to T1P1.SAH at it’s August 10, 2001 meeting in Dallas, Texas and on T1P1/2001-112 R1 which reflected actions taken by the authors to revise and resubmit as per recommendation from T1P1.SAH. The authors advise the readers to please see contribution T1P1/2001-112 R1 for additional reference material on this issue.

Revision 1 of this contribution provides updated information and changes the definition of the ‘triggering event” to be consistent with the FCC 3rd Report and Order.  The contribution clarifies the text and removes ambiguity.

B. Discussion

Background: Clause 4.5, HI2: Interface port for intercept related information, of 3GPP TS 33.108 specifies “Sending of the intercept-related information (IRI) to the LEMF shall in general take place as soon as possible, after the relevant information is available.”  Clause 6.2.1 of 3GPP TS 33.108 V0.1.0 specifies, “As a general principle, within a telecommunication system, intercept related information (IRI), if buffered, should be buffered for as short a time as possible. NOTE:  If the transmission of intercept related information fails, it may be buffered or lost.”

Requirement: Law enforcement agencies need real-time, full-time monitoring capability for interceptions of packet-based communications. Real-time access to IRI refers to the ability to access and monitor communications that occur concurrently with the transmission to or from the interception subject’s equipment, facility, or service.. Access to IRI information generated after service establishment should be provided concurrently with the transmission to or from the interception subject.

Law enforcement agencies need a full-time monitoring capability for interceptions of packet-based communications. The term “full-time” refers to the ability to access and monitor all service activity associated with the interception subject on a 24 hour-per-day basis.

Accurate time-stamps and prompt delivery of intercepted packet-based communications to the monitoring facility are critical to the conduct of law enforcement investigations. The following capabilities address these aspects of LAES.

- Event Time-stamp: Law enforcement agencies need IRI to be time-stamped within a specific amount of time from the receipt by the IAP of the information pertaining to the IRI triggering event . Time-stamping shall be provided for encapsulated interception subject packets delivered to the LEA.

- Event Timing: Law enforcement agencies need IRI sent to the LEMF within a defined amount of time after receipt by the IAP of the information pertaining to the IRI triggering event.

Discussion: The T1.P1 SAH representatives participating on the conference call held on 1/8/2002 agreed to draft timing requirements, which only include access, interception and mediation components, with a demarcation point on the outer boundary of the Delivery Function.  It was decided that the newly drafted requirements would not include the time required to transport the information across a transmission medium of the delivery interface to the LEMF.

CIS/Telcordia took the following action items:

- Reword the requirement with respect to 3.5 seconds to clearly indicate the time is from Access-to Delivery Function (DF) ‘demarcation point’. Identify where to include timing requirements into TS 33.108.

- Review the Time Stamp ASN.1 in TS 33.108 and make recommendations on what needs to be modified/added.  This item has already been addressed in 3GPP and ETSI SEC LI because the time stamp is now imported from ETSI TS 201 671 and the restriction on encoding has been removed.  Therefore Recommendation #3 from T1P1/2002-011 has been removed.

Below find newly drafted timing requirements:

Recommendation #1: Add text in Clause 6.2.1 of TS 33.108 to address the U.S. law enforcement need to have Timing requirements.

Recommendation #2:  Add text to address time-stamps need to be reported to a precision of 1/1000 of a second (i.e., with three decimal places).  This means that TS 33.108 needs to specify the timeStamp parameter must be encoded using the generalizedTime encoding for systems deployed in the U.S.

Proposed Text:

For systems deployed in the U.S., the following timing requirements shall apply to the HI2 “LI application” messages:

- Each IRI data record shall be sent by the delivery function to the LEMF over the HI2 within three and half (3.5) seconds of receipt by the IAP of the information pertaining to the IRI triggering event at least 95% of the time.

- Each IRI data record shall contain a time-stamp that is within 200 milliseconds from the receipt by the IAP of the information pertaining to the IRI triggering event.

- Each IRI data record shall contain a time-stamp reported to 1/1000 of a second  (i.e., with three decimal places) using the GeneralizedTime encoding.

For systems deployed in the U.S., the following timing requirements shall apply to the HI3 CC messages:

- Time-stamps shall be provided for encapsulated interception subject packets delivered to the LEMF.

- Time-stamping procedures shall be consistent with those outlined for the HI2 interface (accuracy and reporting granularity) and shall be encoded using the GeneralizedTime encoding,

C. Recommendation

Approve all recommended changes.