PROJECT:	T1P1 Activity on Lawful Intercept for UMTS

TITLE:	Recommended Changes to T1P1/2001-081 [3GPP TS 33.108 V0.0.2 (2001-06)] to Address U.S.-Specific Lawful Intercept Capabilities for GPRS Packet Domain (Release 99)

SOURCE:	Sandra Lopez

Telcordia Technologies, Inc.

331 Newman Springs Road

Red Bank, NJ 07701

Tel:  	(732) 758-3222

Fax:  	(732) 758-4196

e-mail:	slopez@telcordia.com

DATE:	August 10, 2001

LOCATION: 	Dallas, Texas

ABSTRACT:	This contribution proposes changes to T1P1/2001-081 to address additional U.S.-Specific requirements for Lawful Intercept for GPRS packet domain (Release 99).

DISTRIBUTION:	T1P1, T1P1.SAH

NOTICE

©2001 Telcordia technologies, Inc. The proposals in this submission have been formulated to assist committee T1P1.  This document is offered to the committee as a basis for discussion and is not binding on Telcordia Technologies. The requirements are subject to change in form and numerical value after more study. Telcordia Technologies specifically reserves the right to add to, or amend, the quantitative statements made herein.  Nothing contained herein shall be construed as conferring by implication, estoppel, or otherwise any license or right under any patent, whether or not the use of information herein necessarily employs an invention of any existing or later issued patent.

A. Introduction

This contribution contains recommended changes to the latest version of the 3GPP TS 33.108 specification introduced at the T1P1 meeting held in Ottawa, Canada as contribution T1P1/2001-081.  The proposed changes address editorial concerns and include additional U.S.-Specific requirements for Lawful Interception of General Packet Radio Service (GPRS) packet domain (Release 99).

B. Discussion

The following list summarizes the recommended changes relative to U.S. Lawful Interception requirements under CALEA necessary to enhance the 3GPP TS 33.108 specification to establish a safe harbor for GPRS/R99 UMTS in the U.S..

- Page 5, Introduction, Lines 17 through 21: Changed the Introduction to focus on the agreed upon scope of document which is the specification of the handover interfaces for Intercept Related Information (IRI) and Content of Communication (CC) for the GSM GPRS Packet-Data Domain as defined in Release 99.

- Page 6, Scope, Lines 2 through 8: Changed the Introduction to focus on the agreed upon scope of document which is the specification of the handover interfaces for Intercept Related Information (IRI) and Content of Communication (CC) for the GSM GPRS Packet-Data Domain as defined in Release 99.

- Page 6, Scope, Lines 7-8: HI1, the interface for Administration Functions, is described in Stage 1 and Stage 2 specifications. How does T1P1 plan on addressing administration interface requirements for the U.S.? The statement in this document implies that such effort is expected and is required.

- Page 8, Definitions, symbols, and abbreviations, Lines 35 and 36: Editorial improvements.

- Page 9, Definitions, symbols, and abbreviations, Lines 7 and 9: Added a definition for “communication identifier”, as per text in other sections of the document.

- Page 9, Definitions, symbols, and abbreviations, Line 13 through 16: Expanded the definition of “communication” to explicitly include the U.S. Law enforcement’s definition of communication.

- Page 9, Definitions, symbols, and abbreviations, Lines 17 and 19: Added a definition for “communication identity number”, as per text in other sections of the document.

- Page 9, Definitions, symbols, and abbreviations, Lines 30-31: Deleted the definition of information as it does not appear to be utilized in the document in the context described.

- Page 9, Definitions, symbols, and abbreviations, Lines 42-45: Replaced the word call for communication.

- Page 9-10, Definitions, symbols, and abbreviations, Lines 46-47, 1-2: Enhanced the definition of  interception subject to include U.S. definition for “intercept subject”.

- Page 10, Definitions, symbols, and abbreviations, Lines 8-9: Replaced the word call for communication.

- Page 10, Definitions, symbols, and abbreviations, Lines 21-23: Added a definition for “lawful interception identifier”.

- Page 10, Definitions, symbols, and abbreviations, Lines 31-32: Added a definition for “network element identifier”.

- Page 10, Definitions, symbols, and abbreviations, Lines 33-35: Added a definition for “network identifier”.

- Page 13, Abbreviations, Lines 38, 43, 44: Deleted these abbreviations because they are not used in this document.

- Page 14, Functional requirements, Line 16: Changed “this” for “the”.

- Page 14, Functional requirements, Line 20: Changed “separatly” for “separately”.

- Page 15,  Functional architecture, Line 5: Address the editor’s concern with respect to the administration function and remove the editor’s note.

- Page 15,  Functional architecture, Line 8: Removed the Note because for the US, it might be necessary to support interception in the GGSN.

- Page 15,  Functional architecture, Line 12: Removed the ()’s for clarity and added a comma.

- Page 15,  Functional architecture, Line 13: Replaced “CD” for “IRI”.

- Page 15,  Functional architecture, Line 22: Added the words “internal interception functions”.

- Page 16,  Functional architecture, Line 22: Removed the words “internal interception functions”.

- Page 17,  Handover interface port 3 (HI3), Line 16: Removed the editor’s note.

- Page 17,  Handover interface port 3 (HI3), Line 20: Added a period at the end of the sentence.

- Page 17,  Specific identifiers for LI, Line 27: Removed HI1 from within ().

- Page 17,  Lawful interception identifier (LIID), Line 36: Inserted () around LIID acronym.

- Page 18, Lawful interception identifier (LIID), Line 4: Changed the word “shall” for “may” to address the fact that in the U.S. a single warrant may cover more than one intercept subject identity.  If the word “shall” remains, the administration object model will have to be modified for the U.S. to permit the one to many relationship.

- Page 18, Lawful interception identifier (LIID), Line 8: Removed the editor’s note.

- Page 18,  Communications identifier (CID), Line 11: Added a comma; Line 13: Removed extra space in front of the word “Network”; Lines 15-16: Added a comma and changed order of words around.

- Page 18, Network identifier (NID), Line 31: Removed the editor’s note.

- Page 19, Network identifier (NID), Lines 1-2: Removed the editor’s note.

- Page 19, HI2: interface port intercept related information, Line 11: Added word “as” after “such”.

- Page 19, Data transmission protocols, Line 29: Inserted text to clarify that SMS CC data is intended to be delivered over the HI2 and deleted UUS text as this is circuit-switched  and X.25 related user data.

- Page 20, Types of IRI records, Lines 29-30: Removed word “no” and inserted the word “an” to clarify that LEAs want the IRI-END record upon deactivation of LI.  In addition, the last sentence was removed.

- Page 21, Security aspects, Line 4: Added an “e” to the word “requirments” to spell it correctly as ‘requirements”.  It appears that the editor did not think it was necessary to specify Security requirements for the HI2 and HI3. It is the recommendation of this contribution that the HI2 and HI3 should employ some form of security as the information carried might have to traverse through the shared public infrastructure (e.g., Internet).  It is left for discussion whether such requirements are National requirements or not.

- Page 22, Definition of IRI for packet switched data, Table 2: Added a line in the middle of the table to include the Communication Direction parameter as per ETSI TS 101 509 V8.1.0 (2000-12).

- Page 22, Definition of IRI for packet switched data, Lines 16-17. Changed the word “eight” for “eleven” as per comment # 40 below.

- Page 22, Definition of IRI for packet switched data, Table 1: Added 4 new events to the list.

- Page 22, Definition of IRI for packet switched data: Detailed information about the events and the content of the IRI records (which parameters are included in each record to report each event) to be sent to the LEMF must be included in this specification.  Some material currently available in the ETSI TS 101 509 V8.1.0 might be of use.

- Page 23, Definition of IRI for packet switched data, Table 2: Changed the name of a number of parameters listed in the HI2 ASN.1 parameter column.  Added the intercepted-Call-Direct parameter as it is designed to provide information about whether GPRS PDP Context activation and deactivation events are initiated by the mobile station or the network.  Added the communicationIdentifier parameter because the ASN.1 definition of the SMS-report parameter mandates the inclusion or the communicationIdentifier.  Added the nature-Of-The-intercepted-call parameter because this field indicates SMS or GPRS packet-data is being sent through the HI3 interface.  Added the serverCenterAddress parameter because it identifies the address of the relevant server within the calling (if server is originating) or called (if server is terminating) party address parameters for SMS-MO or SMS-MT. Added the sgsnAddress parameter because it provides the address (IP or X.25) of the SGSN containing the IAP.

- Page 24, Annex A, Line 3: Deleted editor’s note.

- Page 24, Annex A: The list of delivery methods of IRI shall not be limited to the identified methods defined in TS 33.108 Annex A.1 [1]. Other delivery methods shall be allowed for National requirements.  It is the recommendation of this contribution that text should be added to explicitly indicate that additional methods may be identified as National requirements.  Lines 7 and 8 reflect the proposed text.

- Page 24, Annex A: The author of this contribution proposes an additional method of delivery be included in this document.   Since the GSM PLMN GPRS service is an Internet Protocol (IP) domain and GPRS networks support interworking with networks based on the IP, and there is an expressed desire by U.S. Law enforcement to consolidate the number of interfaces law enforcement will need to comply with, it is recommended that a protocol stack of a delivery interface not employ specialized Session and Presentation layer encoding.  Virtual Private Network (VPNs) technology can be employed to provide site-to-site connectivity between the GSM PLMN and the LEMF over a shared public infrastructure for delivering IRI to the LEMF.  IP Tunneling protocols such as IP Security (IPSec) can be used to provide additional security layers (e.g., encryption, authentication, key management) for the IRI packets that are transported to the LEMF. The use of authenticated and encrypted logical links in conjunction with tunneling technology provide a reasonable approach.

- Page 28, Usage of the FTP, Line 23: This statement is in conflict with the LIID definition in Section 7.1 which specifies that the LIID is composed of alphanumeric characters.

- Page 33, Object Tree, Line 5: Removed Editor’s Note, but the content of the note must be taken care of.

- Page 33, HI management operation, Line 45: Removed Editor’s Note, but the content of the note must be taken care of.

- Page 34, Intercept related information (HI2), Line 51: Removed Editor’s Note, but the content of the note must be taken care of.

- Page 46, Intercept related information (HI2),  Parameters: Added pDPContextActivationUnsuccessful, gPRSAttachUnsuccessful, and endOfInterceptionWithPDPContextActive and pDPContextModification codepoints to the GPRSEvent parameter definition to report the 4 new events.

- Page 48, Annex C, Line 1: Changed this Annex to be Normative rather than informative.

- Page 48, Annex C, Line 3: Removed Editor’s Note.

- Page 49, Annex C, Line 20: Changed word “signaling” for “signalling” to be consistent.

- Page 58, Annex F: Add an entry for editorial changes approved in August 10, 2001 T1P1 meeting.

C. Recommendation

Approve all recommended changes.

3GPP TSG SA WG3LI	S3LI01-076

21-23 August, 2001

Saarbrucken, Germany

Source:	Editor

Contact:	Ronald D. (Ron) Ryan

Title: 	3GPP TS 33.108  (Version 0.0.2)

Document for:	Approval

Agenda Item:	_____

NOTICE

©2001 Nortel Networks, Inc.  The proposals in this submission have been formulated to assist  3GPP.   This document is offered to the committee as a basis for  discussion  and  is  not  binding  on  Nortel Networks.  The requirements are subject to change in  form and numerical value after more study.  Nortel Networks  specifically  reserves  the  right  to add to, or amend, the  quantitative statements  made  herein.    Nothing  contained  herein  shall  be  construed  as  conferring by implication,  estoppel, or otherwise any license or right under  any  patent,  whether or not the use of information herein necessarily employs an invention of any  existing  or  later  issued  patent.

A. Introduction

This contribution contains the latest version of 3GPP TS 33.108.

B. Discussion

This version of 3GPP TS 33.108 (V0.0.2) has been created by the editor based on the initial draft contribution and proposal submitted at the Munich meeting (S3LI01-061) and comments on that draft at the meeting. The basis for the text is ETSI TS 201 671 Edition 2 Revision 17.

A draft of this version (V0.0.1), with editorial comments on how the initial TS was constructed, was circulated for comment in June via e-mail. Comments received on that circulated version have been incorporated as noted via e-mail and revision marks and comments solely related to the construction of the TS have been removed in this version.

An attempt has been made to scrub the document of non-GRPS/R99 packet data text and ASN.1.. Per agreement in Munich, the revision marks to the reviewed ASN.1 have been retained in this version. Note that Annex C on GPRS HI3 Interface is marked as informative as specified in ES 201 671. This should be reviewed.

C. Recommendations

Approve as the latest version of TS 33.108.

3GPP TS 33.108 V0.0.2 (2001-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G Security;

Lawful Intercept Handover Interface for Packet Data

(Release 1999)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Security, Lawful Interception, Architecture

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2001, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	11

4	General requirements	13

4.1	Basic principles for the handover interface	13

4.2	Legal requirements	13

4.3	Functional requirements	14

5	Functional architecture	15

6	Overview of handover interface	15

6.1	Handover interface port 2 (HI2)	17

6.2	Handover interface port 3 (HI3)	17

7	Specific identifiers for LI	17

7.1	Lawful interception identifier (LIID)	17

7.2	Communication identifier (CID)	18

7.2.1	Network identifier (NID)	18

7.2.2	Communication identity number (CIN) 	18

7.3	LI correlation between CC and IRI	19

8	HI2: Interface port for intercept related information	19

8.1	Data transmission protocols	19

8.2	Application for IRI (HI2 information)	19

8.3	Types of IRI records	20

9	Performance & quality	20

9.1	Timing	20

9.2	Quality	20

10	Security aspects	21

11	Quantitative aspects	22

12	Definition of IRI for packet switched data	22

Annex A (normative):	HI2 Delivery mechanisms and procedures	24

A.1	ROSE	24

A.1.1	Architecture	24

A.1.2	ASE_HI procedures	25

A.1.2.1	Sending part	25

A.1.2.2	Receiving part	26

A.1.2.3	Data link management	26

A.1.2.3.1	Data link establishment	26

A.1.2.3.2	Data link release	27

A.1.2.4	Handling of Unrecognized Fields and Parameters	27

A.1.3	Profiles	27

A.2	FTP	27

A.2.1	Introduction	27

A.2.2	Usage of the FTP	28

A.2.3	Profiles (this chapter is informative only)	29

A.2.4	File content	30

A.2.5	Exceptional procedures	30

A.2.6	Other Considerations	31

Annex B (normative):	Structure of data at the handover interface	32

B.1	Syntax definitions	32

B.2	Object tree	33

B.3	HI management operation	33

B.4	Intercept related information (HI2)	34

Annex C (informative):	GPRS HI3 Interface	48

C.1	GPRS LI Correlation Header	48

C.1.1	Introduction	48

C.1.2	Definition of GLIC Header	48

C.1.3	Exceptional Procedure	49

C.1.4	Other Considerations	49

C.2	FTP	50

C.2.1	Introduction	50

C.2.2	Usage of the FTP	50

C.2.3	Exceptional procedures	52

C.2.4	CC Contents for FTP	52

C.2.4.1	Fields	52

C.2.4.2	Information Element Syntax	53

C.2.5	Other Considerations	54

Annex D (informative):	LEMF Requirements - Handling of Unrecognized Fields and Parameters	56

Annex E (informative):	Bibliography	57

Annex F (informative):	Change history	58

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This Technical Specification has been produced by the 3GPP TSG SA to allow for the standardization in the area of lawful interception of telecommunications. This document describes the handover interfaces for lawful interception of Packet-Data services within a Third Generation Mobile Communication System (3GMS).  The specification defines the handover interfaces for delivery of lawful interception Intercept Related Information (IRI) and Content of Communication (CC) to the Law Enforcement Monitoring Facility for General Packet Radio Service (GPRS) systems (Release 99).

Laws of individual nations and regional institutions (e.g. European Union), and sometimes licensing and operating conditions define a need to intercept telecommunications traffic and related information in modern telecommunications systems. It has to be noted that lawful interception shall always be done in accordance with the applicable national or regional laws and technical regulations.

# 1	Scope

This specification addresses the handover interfaces for lawful interception of Packet-Data services within a Third Generation Mobile Communication System (3GMS).  The specification defines the handover interfaces for delivery of Intercept Related Information (HI2) and Content of Communication (HI3) to the Law Enforcement Monitoring Facility for GPRS systems. Only the Packet-Data Domain is addressed in this specification. The Circuit-Switched Domain is addressed by different regional specifications.  HI1 is not covered in this specification and is considered to be a matter of national regulation.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 101 331: Telecommunications security; Lawful Interception (LI); requirements of Law Enforcement Agencies

[2]	ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	ITU-T Recommendation X.25: "Interface between Data Terminal Equipment (DTE) and Data Circuit-terminating Equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit".

[5]	EN 300 356-1 to 20: "Integrated Services Digital Network (ISDN); Signaling System No.7; ISDN User Part (ISUP) version 3 for the international interface;  Parts 1 to 20".

[6]	EN 300 403-1 (V1.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

[7]	Void.

[8]	Void.

[9]	Void.

[10]	EN 300 061-1: "Integrated Services Digital Network (ISDN); Subaddressing (SUB) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[11]	Void.

[12]	Void.

[13]	Void.

[14]	EN 300 097-1 including Amendment 1: "Integrated Services Digital Network (ISDN); Connected Line Identification Presentation (COLP) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[15]	EN 300 098-1: "Integrated Services Digital Network (ISDN); Connected Line Identification Restriction (COLR) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[16]	EN 300 130-1: "Integrated Services Digital Network (ISDN); Malicious Call Identification (MCID) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[17]	EN 300 138-1 including Amendment 1: "Integrated Services Digital Network (ISDN); Closed User Group (CUG) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[18]	Void.

[19]	EN 300 185-1: "Integrated Services Digital Network (ISDN); Conference call, add-on (CONF) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[20]	ETS 300 188-1: "Integrated Services Digital Network (ISDN); Three-Party (3PTY) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[21]	EN 300 207-1 (V1.2): "Integrated Services Digital Network (ISDN); Diversion supplementary services; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[22]	Void.

[23]	EN 300 286-1: "Integrated Services Digital Network (ISDN); User-to-User Signalling (UUS) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[24]	Void.

[25]	EN 300 369-1 (V1.2): "Integrated Services Digital Network (ISDN); Explicit Call Transfer (ECT) supplementary service; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[26]	Void.

[27]	Void.

[28]	Void.

[29]	EN 300 196-1 (V1.2): "Integrated Services Digital Network (ISDN); Generic functional protocol for the support of supplementary services; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[30]	Void.

[31]	ITU-T Recommendation Q.850: "Usage of cause and location in the Digital Subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN User Part".

[32]	GSM 09.02: "Digital cellular telecommunications system (Phase 2+); Mobile Application Part (MAP) specification".

[33]	ITU-T Recommendation X.680: "Specification of Abstract Syntax Notation One (ASN.1)".

[34]	ITU-T Recommendation X.690: "Specification of basic encoding rules for Abstract Syntax Notation One (ASN.1)".

[35]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[36]	ITU-T Recommendation X.881: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) service definition".

[37]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) protocol specification".

[38]	Void.

[39]	EN 300 122-1: "Integrated Services Digital Network (ISDN); Generic keypad protocol for the support of supplementary services; Digital Subscriber Signalling System No. one (DSS1) protocol; Part 1: Protocol specification".

[40]	ETS 300 392-1: "Terrestrial Trunked Radio (TETRA); Voice plus Data (V+D); Part 1: General network design".

[41]	EN 300 940, GSM 04.08: " Digital cellular communications system (Phase 2+) ; Mobile radio interface layer 3 specification ".

[42]	TS 101 509 "Digital cellular telecommunications system (Phase 2+);
Lawful interception; Stage 2 (GSM 03.33)

[43]	GSM 03.03: “Digital cellular telecommunications system (Phase 2+); Numbering, addressing and identification”.

[44]	EN 301 344, GSM 03.60: “Digital cellular telecommunications system (Phase 2+); GPRS Service description stage 2”.

[45]	GSM 09.60 (EN 301 347): "Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); GPRS tunelling protocol (GTP) across Gn and Gp Interface

[46]	STD 9 "File Transfer Protocol (FTP)", October 1985

[47]	RFC2228 "FTP Security Extensions", October 1997

[48]	ITU-T recommendation Q.763 Signalling System No.7 - ISDN User Part formats and codes

[49]	GSM 12.15 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging & Billing; GSM call and event data for the Packet Switched (PS) domain) ”

[50]	void

[51]	STD0005 "Internet Protocol"

[52]	STD0007 “Transmission Control Protocol”

[53]	TR 101 876 “Telecommunications security; Lawful Interception (LI); Description of GPRS HI3”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

access provider: access provider provides a user of some network with access from the user's terminal to that network.

NOTE 1:	This definition applies specifically for the present document. In a particular case, the access provider and network operator may be a common commercial entity.

(to) buffer: temporary storing of information in case the necessary telecommunication connection to transport information to the LEMF is temporarily unavailable.

communication identifier:  identifier generated to identify each activity of a target identity.  This identifier can also be used for correlation between IRI records and CC connections.

.

communication: Information transfer according to agreed conventions. Intelligence of any nature transmitted by any method.  Any transfer of messages, signals, writing, images, sounds, data, or intelligence of any nature transmitted in whole or in part by a wire, radio, electromagnetic, photoelectric, or photo-optical system, etc.  It encompasses the term “electronic communications, and “wire communications”.

communication identity number:  temporary identifier of an intercepted communication, relating to a specific target identity, when reporting events for connection-oriented types of communication (e.g., circuit-switched calls).

content of communication: information exchanged between two or more users of a telecommunications service, excluding intercept related information. This includes information which may, as part of some telecommunications service, be stored by one user for subsequent retrieval by another.

handover interface: physical and logical interface across which the interception measures are requested from network operator / access provider / service provider, and the results of interception are delivered from a network operator / access provider / service provider to a law enforcement monitoring facility.

identity: technical label which may represent the origin or destination of any telecommunications traffic, as a rule clearly identified by a physical telecommunications identity number (such as a telephone number) or the logical or virtual telecommunications identity number (such as a personal number) which the subscriber can assign to a physical access on a case-by-case basis.

interception: action (based on the law), performed by an network operator / access provider / service provider, of making available certain information and providing that information to a law enforcement monitoring facility.

NOTE 2:	In the present document the term interception is not used to describe the action of observing communications by a law enforcement agency.

interception configuration information: information related to the configuration of interception.

Interception interface: physical and logical locations within the network operator's / access provider's / service provider's telecommunications facilities where access to the content of communication and intercept related information is provided. The interception interface is not necessarily a single, fixed point.

interception measure: technical measure which facilitates the interception of telecommunications traffic pursuant to the relevant national laws and regulations.

intercept related information: collection of information or data associated with telecommunication services involving the target identity, specifically communication associated information or data (e.g. unsuccessful communication attempts), service associated information or data (e.g. service profile management by subscriber) and location information.

interception subject: person or persons, specified in a lawful authorization, whose telecommunications equipment, facilities, or services are subject to be lawful interception.

internal intercepting function: point within a network or network element at which the content of communication and the intercept related information are made available.

internal network interface: network's internal interface between the Internal Intercepting Function and a mediation device.

invocation and operation: describes the action and conditions under which the service is brought into operation; in the case of a lawful interception this may only be on a particular communication. It should be noted that when lawful interception is activated, it shall be invoked on all communications (Invocation takes place either subsequent to or simultaneously with activation.). Operation is the procedure which occurs once a service has been invoked.

NOTE 3:	The definition is based on [37], but has been adapted for the special application of lawful interception, instead of supplementary services.

law enforcement agency: organization authorized by a lawful authorization based on a national law to request interception measures and to receive the results of telecommunications interceptions.

law enforcement monitoring facility: law enforcement facility designated as the transmission destination for the results of interception relating to a particular interception subject.

lawful authorization: permission granted to a LEA under certain conditions to intercept specified telecommunications and requiring co-operation from a network operator / access provider / service provider. Typically this refers to a warrant or order issued by a lawfully authorized body.

lawful interception: see interception.

lawful interception identifier: identifier designated by law enforcement for a particular surveillance and provided to a telecommunications service provider at the time of provisioning of a surveillance.

location information: information relating to the geographic, physical or logical location of an identity relating to an interception subject.

mediation device: equipment, which realizes the mediation function.

mediation function: mechanism which passes information between a network operator, an access provider or service provider and a handover interface, and information between the internal network interface and the handover interface.

network element: component of the network structure, such as a local exchange, higher order switch or service control processor.

network element identifier:  [Editor note: need a definition here] uniquely identifies the relevant network element carrying out the lawful interception.

network identifier:  internationally unique identifier that includes a unique identification of the network operator, access provider, or service provider and/or unique identifier of the relevant network element carrying out the lawful interception.

network operator: operator of a public telecommunications infrastructure which permits the conveyance of signals between defined network termination points by wire, by microwave, by optical means or by other electromagnetic means.

quality of service: quality specification of a telecommunications channel, system, virtual channel, computer-telecommunications session, etc. Quality of service may be measured, for example, in terms of signal-to-noise ratio, bit error rate, message throughput rate or call blocking probability.

reliability: probability that a system or service will perform in a satisfactory manner for a given period of time when used under specific operating conditions.

result of interception: information relating to a target service, including the content of communication and intercept related information, which is passed by a network operator, an access provider or a service provider to a law enforcement agency. Intercept related information shall be provided whether or not call activity is taking place.

service information: information used by the telecommunications infrastructure in the establishment and operation of a network related service or services. The information may be established by a network operator, an access provider, a service provider or a network user.

service provider: natural or legal person providing one or more public telecommunications services whose provision consists wholly or partly in the transmission and routing of signals on a telecommunications network. A service provider needs not necessarily run his own network.

SMS: Short Message Service gives the ability to send character messages to phones. SMS messages can be MO (mobile originate) or MT(mobile terminate).

target identity: technical identity (e.g. the interception's subject directory number), which uniquely identifies a target of interception. One target may have one or several target identities.

target service: telecommunications service associated with an interception subject and usually specified in a lawful authorization for interception.

NOTE 4:	There may be more than one target service associated with a single interception subject.

telecommunications: any transfer of signs, signals, writing images, sounds, data or intelligence of any nature transmitted in whole or in part by a wire, radio, electromagnetic, photoelectronic or photo-optical system.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3PTY	Three-Party Service

AA	Abbreviated Address

AC	Alarm Call

ACM	Address Complete Message

AOC	Advice of Charge Service

AP	Access Provider

ASN.1	Abstract Syntax Notation, Version 1

ASE	Application Service Element

ATM	Asynchronous Transfer Mode

BA	DSS1 Basic Access

BC	Bearer Capability

BCSM	Basic Call State Model

BER	Basic Encoding Rules

BS	Basic Service

CC	Content of Communication

CCBS	Completion of Calls to Busy Subscriber

CCF	Call Control Function

CCNR	Completion of Calls on No Reply

CD	Call Deflection

CF	Call Forwarding

CFB	Call Forwarding on Busy

CFNR	Call Forwarding on No Reply

CFU	Call Forwarding Unconditional

CH	Call Hold

CCLID	CC Link Identifier

CID	Communication Identifier

CIN	Call Identity Number

CLI	Calling Line Identity (Calling Party Number)

CLIP	Calling Line Identification Presentation

CLIR	Calling Line Identification Restriction

COL	Connected Line Identity (Connected Number)

COLP	Connected Line Identification Presentation

COLR	Connected Line Identification Restriction

CONF	Conference Call, Add-on

CPG	Call Progress Message

CPH	Call Party Handling

CSi	Capability Set 'i'

CUG	Closed User Group

CUSF	Call Unrelated Service Function

CW	Call Waiting

DDI 	Direct Dialing In

DF	Delivery Function

DIV	Call Diversion Services

DN	Directory Number

DSS1	Digital Subscriber Signalling system No.1

DTMF 	Dual Tone Multi-Frequency

ECT	Explicit Call Transfer

FB	Fallback Procedure

FDC	Fixed Destination Call

FPH	Freephone

FTP	File Transfer Protocol

GGSN	Gateway GPRS Support Node

GLIC	GPRS LI Correlation

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

GSN	GPRS Support Node (SGSN or GGSN)

GTP	GPRS Tunnelling Protocol

HI	Handover Interface

HI1	Handover Interface Port 1 (for Administrative Information)

HI2	Handover Interface Port 2 (for Intercept Related Information)

HI3	Handover Interface Port 3 (for Content of Communication)

HLC	High Layer Compatibility

HOLD	Call Hold Service

IA	Interception Area

IA5	International Alphabet No. 5

IAM	Initial Address Message

IAP	Interception Access Point

ICB	Incoming Call Barring

ICC	Interception Control Centre

ICI	Interception Configuration Information

IE	Information Element

IIF	Internal Interception Function

IMEI	International Mobile station Equipment Identity

IMSI	International Mobile Subscriber Identity

IN	Intelligent Network

INAP	Intelligent Network Application Part

INI	Internal network interface

IP	Internet Protocol

IPS	Internet Protocol Stack

IRI	Intercept Related Information

ISDN	Integrated services digital network

ISUP	ISDN user part

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LIID	Lawful Interception Identifier

LLC	Lower layer compatibility

LSB	Least significant bit

MAP	Mobile Application Part

MCID	Malicious Call Identification

MF	Mediation Function

MMC	Meet-me Conference

MS	Mobile Station

MSB	Most significant bit

MSISDN	Mobile Subscriber ISDN Number

MSN	Multiple Subscriber Number

NDUB	Network Determined User Busy

NEID	Network Element Identifier

NID	Network Identifier

NWO	Network Operator

OA&M	Operation, Administration & Maintenance

OCB	Outgoing Call Barring

PDP	Packet Data Protocol

PLMN	Public land mobile network

PR	Partial Rerouting

PRA	ISDN Primary Rate Access

PSTN	Public Switched Telephone Network

ROSE	Remote Operation Service Element

Rx	Receive direction

SCI	Subscriber Controlled Input

SCF	Service Control Function

SCP	Service Control Point

SDF	Service Data Function

SGSN	Serving GPRS Support Node

SMAF	Service Management Agent Function

SMF	Service Management Function

SMS	Short Message Service

SPC	Signalling Point Code

SRF	Specialized Resource Function

SS	Supplementary Service

SS No.7	Common Channel Signalling System ITU(T) No. 7

SSF	Service Switching Function

SSP	Service Switching Point

STC	Sub-Technical Committee

STUI	Service To User Information

SUB	Subaddressing Supplementary Service

SvP	Service Provider

TCP	Transmission Control Protocol

TE	Target Exchange

TETRA	Trans European Trunked Radio

TI	Target identity

TMR	Transmission Medium Requirement

TP	Terminal Portability

T-PDU		tunneled PDU

Tx	Transmit direction

UI	User Interaction

UMTS	Universal Mobile Telecommunication System

USI	User Service Information

UTSI	User To Service Information

VPN	Virtual Private Network

xGSN	SGSN or GGSN

# 4	General requirements

The present document focuses on the handover interface related to the provision of information related to LI between a network operator, access provider and/or service provider and a Law Enforcement Agency (LEA).

## 4.1	Basic principles for the handover interface

The network requirements mentioned in the present document are derived, in part, from the requirements defined in ES 201 158 [2].

Lawful interception requires functions to be provided in some, or all of, the switching or routing nodes of a telecommunications network.

The specification of the handover interface is subdivided into three ports each optimised to the different purposes and types of information being exchanged.

- The interface is extensible.

## 4.2	Legal requirements

It shall be possible to select elements from the handover interface specification to conform with:

-	national requirements;

-	national law;

-	any law applicable to a specific LEA.

As a consequence, the present document shall define, in addition to mandatory requirements, which are always applicable, supplementary options, in order to take into account the various influences listed above . See also [1] and [3].

## 4.3	Functional requirements

A lawful authorization shall describe the kind of information (Content of Communication (CC) and/or Intercept Related Information (IRI)) that is required by the LEA, the interception subject, the start and stop time of LI, and the addresses of the LEAs for CC and/or IRI and further information.

A single interception subject may be the subject to interception by different LEAs. It shall be possible strictly to separate these interception measures.

If two targets are communicating with each other, each target is dealt with separately.

# 5	Functional architecture

Figure 1 is the reference configuration for lawful interception. There is one Administration Function (ADMF) in the network. Together with the delivery functions it is used to hide from the xGSN that there might be multiple activation’s by different Law Enforcement Agencies (LEMFs) on the same target.

Figure 1: Reference configuration

The reference configuration is only a logical representation of the entities involved in lawful interception and does not mandate separate physical entities. This allows for higher levels of integration.

A communication could be intercepted based on several identities (MSISDN, IMSI, IMEI) of the same target.

For the delivery of the CC and IRI, the xGSN provides a correlation number and target identity to the DF2 and DF3 which is used there to select the different LEMFs where the CC/IRI shall be delivered to.

# 6	Overview of handover interface

The generic handover interface adopts a three port structure such that administrative information (HI1), intercept related information (HI2) and the content of communication (HI3) are logically separated.

Figure 2 shows a block diagram with the relevant entities for Lawful Interception.

The outer circle represents the NWO/AP/SvP´s domain with respect to lawful interception. It contains the network internal functions, the internal network interface (INI), the administration function and the mediation functions for IRI and CC. The inner circle contains the internal functions of the network (e.g. switching, routing, handling of the communication process). Within the network internal function the results of interception (IRI, CC) are generated in the internal interception functions (IIF).

The IIF provides the content of communication (CC) and the intercept related information (IRI), respectively, at the internal network interface (INI). For both kinds of information, mediation functions may be used, which provide the final representation of the standardized handover interfaces at the NWO/AP/SvP's domain boundary.

Figure 2: Functional block diagram showing handover interface HI

NOTE 1:	Figure 2 shows only a reference configuration, with a logical representation of the entities involved in lawful interception and does not mandate separate physical entities.

NOTE 2:	The mediation functions may be transparent.

## 6.1	Handover interface port 2 (HI2)

- The handover interface port 2 shall transport the IRI from the NWO/AP/SvP's IIF to the LEMF.

The delivery shall be performed via data communication methods which are suitable for the network infrastructure and for the kind and volume of data to be transmitted.

The delivery can in principle be made via different types of lower communication layers, which should be standard or widely used data communication protocols.

The individual IRI parameters shall be coded using ASN.1 and the basic encoding rules (BER). The format of the parameter's information content shall be based on existing telecommunication standards, where possible.

The individual IRI parameters have to be sent to the LEMF at least once (if available).

The IRI records shall contain information available from normal network or service operating procedures. In addition the IRI records shall include information for identification and control purposes as specifically required by the HI2 port.

The IIF is not required to make any attempt to request explicitly extra information which has not already been supplied by a signalling system.

## 6.2	Handover interface port 3 (HI3)

The port HI3 shall transport the content of the communication (CC) of the intercepted telecommunication service to the LEMF. The content of communication shall be presented as a transparent en-clair copy of the information flow during an established, frequently bi-directional, communication of the interception subject.

- As the appropriate form of  HI3 depends upon the service being intercepted, HI3 is described in relevant annexes.

- The HI2 and HI3 are logically different interfaces, even though in some installations the HI2 and HI3 packet streams might also be delivered via a common transmission path from a MF to a LEMF. It is possible to correlate HI2 and HI3 packet streams by having common (referencing) data fields embedded in the IRI and the CC packet streams.

# 7	Specific identifiers for LI

Specific identifiers are necessary to identify a target for interception uniquely and to correlate between the data, which is conveyed over the different handover interfaces (HI2 and HI3). The identifiers, which apply to all communication technologies, are defined in the subsections below. Additional technology specific identifiers are defined in related annexes.

## 7.1	Lawful interception identifier (LIID)

For each target identity related to an interception measure, the authorized NWO/AP/SvP operator shall assign a special lawful interception identifier (LIID), which has been agreed between the LEA and the NWO/AP/SvP.

Using an indirect identification, pointing to a target identity makes it easier to keep the knowledge about a specific interception target limited within the authorized NWO/AP/SvP operators and the handling agents at the LEA.

The lawful interception identifier (LIID) is a component of the CC delivery procedure and of the IRI records. It shall be used within any information exchanged at the handover interfaces HI2 and HI3 for identification and correlation purposes.

The LIID format shall consist of alphanumeric characters. It might for example, among other information, contain a lawful authorization reference number, and the date, when the lawful authorization was issued.

The authorized NWO/AP/SvP may enter for each target identity of the interception subject a unique LIID.

Example:	The interception subject has an ISDN access with three MSNs. The NWO/AP/SvP enters for each MSN an own LIID.

If more than one LEA intercepts the same target identity, there shall be unique LIIDs assigned, relating to each LEA.

Warrant reference number [42]	  Lawful interception identifier (LIID)

## 7.2	Communication identifier (CID)

For each communication or other activity relating to a target identity, a CID is generated by the relevant network element. The CID consists of the following two identifiers:

-	Network identifier (NID);

-	Communication Identity Number (CIN).

For the communication identifier (CID) in the GPRS system,  a combination of GGSN address and charging ID is used.

NOTE:	If interception has been activated for both parties of the packet data communication both CC and IRI will be delivered for each party as separate intercept activity.

### 7.2.1	Network identifier (NID)

The network identifier is a mandatory parameter; it should be internationally unique. It consists in one or both of  the following two identifiers. It is mandatory that one of them is used.

1)	NWO/AP/SvP- identifier (optional):
Unique identification of network operator, access provider or service provider.

2)	Network element identifier NEID (optional):
The purpose of the network element identifier is to uniquely identify the relevant network element carrying out the LI operations, such as LI activation, IRI record sending, etc.

A network element identifier may be:

-	an IP address.

xGSN address [42]    Network identifier (NID)

### 7.2.2	Communication identity number (CIN)

The communication identity number is a temporary identifier of an intercepted communication, relating to a specific target identity.

## 7.3	LI correlation between CC and IRI

For the delivery of CC and IRI, the SGSN or GGSN provides correlation numbers and target identities to the HI2 and HI3. The correlation number is unique per PDP context and is used to correlate CC with IRI and the different IRI’s of one PDP context.

# 8	HI2: Interface port for intercept related information

The HI2 interface port shall be used to transport all intercept-related information (IRI), i.e. the information or data associated with the communication services of the target identity apparent to the network. It includes signalling information used to establish the telecommunication service and to control its progress, time stamps, and, if available, further information such as location information. Only information which is part of standard network signalling procedures shall be used within communication related IRI.

Sending of the intercept-related information (IRI) to the LEMF shall in general take place as soon as possible, after the relevant information is available.

In exceptional cases (e.g. data link failure), the intercept related information may be buffered for later transmission for a specified period of time.

Within this section only definitions are made which apply in general for all network technologies. Additional technology specific HI2 definitions are specified in related Annexes.

## 8.1	Data transmission protocols

The protocol used by the "LI application" for the encoding and the sending of data between the MF and the LEMF is based on already standardized data transmission protocols like ROSE or FTP.

The specified data communication methods provide a general means of data communication between the LEA and the NWO/AP/SvP's mediation function. They are used for the delivery of:

-	HI2 type of information (IRI records);

-	HI3 data type of information over the HI2 in certain circumstances (SMS, etc.).

The present document specifies the use of the two possible methods for delivery: ROSE or FTP on the application layer and the BER on the presentation layer. The lower layers for data communication may be chosen in agreement with the NWO/AP/SvP and the LEA.

The delivery to the LEMF should use the internet protocol stack.

## 8.2	Application for IRI (HI2 information)

The handover interface port 2 shall transport the intercept related information (IRI) from the NWO/AP/SvP's MF to the LEMF.

The individual IRI parameters shall be coded using ASN.1 and the basic encoding rules (BER). Where possible, the format of the information content shall be taken over from existing telecommunication standards, which are used for these parameters with the network already (e.g. the ISDN user part, DSS1, MAP and IP). Within the ASN.1 coding for IRI, such standard parameters are typically defined as octet strings.

## 8.3	Types of IRI records

Intercept related information shall be conveyed to the LEMF in messages, or IRI data records, respectively. Four types of IRI records are defined:

1)	IRI-Begin record 			at the first event of a communication attempt,
								opening the IRI transaction

2)	IRI-End record 				at the end of a communication attempt,
								closing the IRI transaction

3)	IRI-Continue record 		at any time during a communication attempt
								within the IRI transaction

4)	IRI-REPORT record 		used in general for non-communication related events

For information related to an existing communication case, the record types 1 to 3 shall be used. They form an IRI transaction for each communication case or communication attempt, which corresponds directly to the communication phase (set-up, active or release).

For some packet oriented data services such as GPRS,  the first event of a communication attempt shall be the PDP context activation or a similar event and an IRI-BEGIN record shall be issued. The end of the communication attempt shall be the PDP context deactivation or a similar event and an IRI-END record shall be issued. While a PDP context is active, IRI-CONTINUE records shall be used for CC relevant IRI data records, IRI-REPORT records otherwise.

Record type 4 is used for non-communication related subscriber action, like subscriber controlled input (SCI) for service activation. For simple cases, it can also be applicable for reporting unsuccessful communication attempts.

The record type is an explicit part of the record. The 4 record types are defined independently of target communication events. The actual indication of one or several communication events, which caused the generation of an IRI record, is part of further parameters within the record's, information content. Consequently, the record types of the IRI transactions are not related to specific messages of the signaling protocols of a communication case, and are therefore independent of future enhancements of the intercepted services, of network specific features, etc. Any transport level information (i.e.  higher-level services) on the target communication-state or other target communication related information is contained within the information content of the IRI records.

For some packet oriented data services such as GPRS, if LI is being activated during an already established PDP context or similar, an IRI-BEGIN record will mark the start of the interception. If LI is being deactivated during an established PDP context or similar, an IRI-END record will be transmitted.

# 9	Performance & quality

## 9.1	Timing

As a general principle, within a telecommunication system, intercept related information (IRI), if buffered, should be buffered for as short a time as possible.

NOTE:	If the transmission of intercept related information fails, it may be buffered or lost.

## 9.2	Quality

The quality of service associated with the result of interception should be (at least) equal to the quality of service of the original content of communication.

# 10	Security aspects

Security is defined by national requirements.

# 11	Quantitative aspects

- The number of target interceptions supported is a national requirement.

# 12	Definition of IRI for packet switched data

Intercept related information will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (set up of a data context, target may be the originating or terminating party).

2.	At the end of a connection, when the target identity becomes inactive (removal of a data context).

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2, e.g. information on subscriber controlled input.

The intercept related information (IRI) may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information).

2.	Basic data context information, for standard data transmission between two parties.

- The events defined in ref [43] are used to generate records for the delivery via HI2.

There are eleven different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table give the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 1: Mapping between GPRS Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| GPRS attach (successful) | REPORT |
| GPRS attach (unsuccessful) | REPORT |
| GPRS detach | REPORT |
| PDP context activation (successful) | BEGIN |
| PDP context activation (unsuccessful) | REPORT |
| Start of intercept with PDP context active | BEGIN |
| PDP context deactivation | END |
| End of intercept with PDP context active | END |
| Cell and /or RA update | REPORT if no PDP context is active CONTINUE if, at least, one PDP context is active |
| PDP context modification | CONTINUE |
| SMS | REPORT |

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in xGSN or DF2/MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Table 2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation/msISDN |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation/imsi |
| observed IMEI | Target Identifier with the IMEI of the target subscriber (monitored subscriber) | partyInformation/imei |
| observed PDP address | PDP address used by the target.. | partyInformation/pDP-address-allocated-to-the-target |
| event type | Description which type of event is delivered: PDP Context Activation, PDP Context Deactivation,GPRS Attach, etc. | gPRSevent |
| event date | Date of the event generation in the xGSN | timeStamp |
| event time | Time of the event generation in the xGSN |  |
| Access Point Name | The APN of the access point | partyInformation/aPN |
| PDP type | This field describes the PDP type as defined in TS GSM 09.60, TS GSM 04.08, TS GSM 09.02 | partyInformation/pDP-type |
| communication direction | This field indicates whether the PDP context activation or deactivation is MS directed or network initiated. | intercepted-Call-Direct |
| communication identifier | This field is used to uniquely identify an intercepted communication. | communicationIdentifier |
| Nature of intercepted communication | This field indicates SMS or GPRS packet-data is being sent through the HI3 interface. | nature-Of-The-intercepted-call |
| CID | Unique number for each PDP context delivered to the LEMF , to help the LEA, to have a correlation between each PDP Context and the IRI. | gPRSCorrelationNumber |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| CGI (Cell Global ID) | Cell number of the target; for the location information | locationOfTheTarget/globalCellId |
| routing area code | Routing-area-code of the target defines the Routing Area in a GPRS-PLMN | locationOfTheTarget/rAId |
| SMS | The SMS content with header which is sent with the SMS-service | sMS |
| failed context activation reason | This field gives information about the reason(s) for failed context(s) activation of the target subscriber. | gPRSOperationErrorCode |
| failed attach reason | This field gives information about the reason(s) for failed attach attempts of the target subscriber. | GPRSOperationErrorCode |
| SGSN Address | This field provides the address (IP or X.25) of the SGSN containing the IAP. | sgsnAddress |
| Server center address | This field identifies the address of the relevant server within the calling (if server is originating) or called (if server is terminating) party address parameters for SMS-MO or SMS-MT. | serverCenterAddress |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

Annex A (normative):
HI2 Delivery mechanisms and procedures

There are two possible methods for delivery of IRI to the LEMF:

a)	ROSE

b)	FTP

According to national requirements additional methods for delivery may be identified.  If no additional methods are identified as national requirements, at least one of these methods shall be provided.

# A.1	ROSE

## A.1.1	Architecture

Figure A.1: Architecture

The ASE_HI manages the data link, the coding/decoding of the ROSE operations and the sending/receiving of the ROSE operations.

## A.1.2	ASE_HI procedures

### A.1.2.1	Sending part

To request the sending of data to a peer entity, the LI_Application provides the ASE_HI, the address of the peer entity, the nature of the data and the data.

On receiving a request of the LI_Application:

-	if the data link toward the peer entity address is active, the ASE_HI, from the nature of the data provided, encapsulates this data in the relevant RO-Invoke operation.

-	if the data link toward the peer entity address isn't active, the ASE_HI establishes this data link (see annex A.1.2.3). Then, depending on the nature of the data provided, the ASE_HI encapsulates this data in the relevant RO-Invoke operation.

- Depending on the natures of the data provided by the LI_Application, the ASE_HI encapsulates this data within the relevant ROSE operation:

-

-	IRI: in this case the data provided by the application are encoded within the class 2 RO-Invoke operation Sending_of_IRI.

-	SMS: in this case the data provided by the application are encoded within the class 2 RO-Invoke operation Sending-of-IRI.

- Depending on the class of the operation, the ASE-HI may have to wait for an answer. In this case a timer, depending on the operation, is started on the sending of the operation and stopped on the receipt of an answer (RO_Result, RO_Error, RO_Reject).

- On timeout of the timer, the ASE_HI indicates to the LI_Application that no answer has been received. It is under the LI_Application responsability to send again the data or to inform the administrator of the problem.

On receipt of an answer component (after verification that the component isn't erroneous), the ASE_HI stop the relevant timer and acts depending on the type of component:

-	on receipt of a RO_Result, the ASE_HI provide the relevant LI_Application an indication that the data has been received by the peer LI-application and the possible parameters contained in the RO_Result.

-	on receipt of a RO_Error, the ASE_HI provide the relevant LI_Application an indication that the data hasn't been received by the peer LI-application and the possible "Error cause". The error causes are defined for each operation in the relevant ASN1 script. It is under the LI_Application responsability to generate or not an alarm message toward an operator or administrator.

-	on receipt of a RO_Reject_U/P, the ASE_HI provide the relevant LI_Application an indication that the data hasn't been received by the peer LI-application and the "Problem cause". The "problem causes" are defined in [35] to [37]. It is under the LI_Application responsibility to send again the data or to inform the operator/administrator of the error.

- On receipt of an erroneous component, the ASE_HI acts as described in ITU-T Recommendations [35] to [37].

### A.1.2.2	Receiving part

- On receipt of a ROSE operation from the lower layers:

-	When receiving operations from the peer entity, the ASE_HI verifies the syntax of the component and transmits the parameters to the LI-Application. If no error/problem is detected, in accordance with the [35] to [37] standard result (only Class2 operation are defined), the ASE_HI sends back a RO_Result which coding is determined by the relevant operation ASN1 script. The different operations which can be received are:

-

-	RO-Invoke operation "Sending-of-IRI" (HI2 interface);

-

-	RO-Invoke operation "No-Circuit-Call-Related-Services" (HI3 interface);

-

- In case of error, the ASE_HI acts depending on the reason of the error or problem:

-	in accordance with the rules defined by [35] to [37], an RO_Error is sent in case of unsuccessfully operation at the application level. The Error cause provided is one among those defined by the ASN1 script of the relevant operation.

-	in accordance with the rules defined in [35] to [37], an RO_Reject_U/P is sent in case of erroneous component. On receipt of an erroneous component, the ASE_HI acts as described in [35] to [37].

### A.1.2.3	Data link management

- This function is used to establish or release a data link between two peer LI_Applications entities (MF and LEMF).

- Depending on a per destination address configuration data, the data link establishment may be required either by the LEMF LI_Application or by the MF LI_Application.

#### A.1.2.3.1	Data link establishment

- To request the establishment of a data link toward a peer entity, the LI_Application provides, among others, the destination address of the peer entity (implicitly, this address defined the protocol layers immediately under the ASE_HI: TCP/IP, X25, …). On receipt of this request, the ASE_HI request the establishment of the data link with respect of the rules of the under layers protocol.

- As soon as the data link is established, the requesting LI_Application initiates an authentication procedure:

-	the origin LI_Application requests the ASE_HI to send the class 2 RO-Invoke operation "Sending_of_Password" which includes the "origin password" provided by the LI_Application;

-	the peer LI-Application, on receipt of the "origin password" and after acceptance, requests to its ASE_HI to send back a RO-Result. In addition, this destination application requests the ASE_HI to send the class 2 RO-Invoke operation "Sending-of-Password" which includes the "destination password" provided by the LI_Application;

-	the origin LI-Application, on receipt of the "destination password" and after acceptance, requests to its ASE_HI to send back a RO-Result. This application is allowed to send data;

-	after receipt of the RO_Result, this application is allowed to send data.

- In case of erroneous password, the data link is immediately released and an "password error indication" is sent toward the operator.

- Optionally a Data link test procedure may be used to verify periodically the data link:

-	When no data have been exchanged during a network dependent period of time toward an address, (may vary from 1 to 30 minutes) the LI_Application requests the ASE_HI to send the class 2 RO-Invoke operation Data-Link-Test.

-	The peer LI-Application, on receipt of this operation , requests to it's ASE_HI to send back a RO-Result.

-	On receipt of the Result the test is considered valid by the LI_Application.

-	If no Result is received or if a Reject/Error message is received, the LI_Aplication requests the ASE_LI to release the data link and send an error message toward the operator.

#### A.1.2.3.2	Data link release

-	The End of the connection toward the peer LI_Application is under responsibility of the LI_Application. E.g, the End of the connection may be requested in the following cases:

-	When all the data (IRI, …) has been sent. To prevent unnecessary release, the datalink may be released only when no LI_Application data have been exchanged during a network dependent period of time.

-	The data link is established when a call is intercepted and released when the intercepted call is released (and all the relevant data have been sent).

-	For security purposes.

-	For changing of password or address of the LEMF/IIF.

-	Etc.

-	To end the connection an LI_Application requests the ASE_HI to send the class 2 RO-Invoke operation "End-Of-Connection".

-	The peer LI-Application, on receipt of this operation , requests to it's ASE_HI to send back a RO_Result.

-	On receipt of the Result the LI_Application requests the ASE_LI to release the data link.

-	If no Result is received after a network dependent period of time, or if a Reject/Error message is received, the LI_Application requests the ASE_LI to release the data link and to send an error message toward the operator/administrator.

#### A.1.2.4	Handling of Unrecognized Fields and Parameters

See annex D.

## A.1.3	Profiles

Not covered in this edition.

# A.2	FTP

## A.2.1	Introduction

At HI2 interface FTP is used over internet protocol stack for the delivery of the IRI. The FTP is defined in ref [46]. The IP is defined in ref [51]. The TCP is defined in ref [52].

FTP supports reliable delivery of data. The data may be temporarily buffered in the mediation function (MF) in case of link failure. FTP is independent of the payload data it carries.

## A.2.2	Usage of the FTP

The MF acts as the FTP client and the LEMF acts as the FTP server . The client pushes the data to the server.

The receiving node LEMF stores the received data as files. The MF may buffer files.

Several records may be gathered to bigger packages prior to sending, to increase bandwidth efficiency.

The following configurable intercept data collection (= transfer package closing / file change) threshold parameters should be supported:

-	frequency of transfer, based on send timeout, e.g. X ms

-	frequency of transfer, based on volume trigger, e.g. X octets

Every file shall contain only complete IRI records. The single IRI record shall not be divided into several files.

There are two possible ways how the interception data may be sent from the MF to the LEMF. One way is to produce files that contain interception data only for one observed target (ref: "File naming method A)"). The other way is to multiplex all the intercepted data that MF receives to the same sequence of general purpose interception files sent by the MF (ref: "File naming method B)").

File naming:

The names for the files transferred to a LEA are formed according to one of the 2 available formats, depending on the delivery file strategy chosen (e.g. due to national convention or operator preference).

Either each file contains data of only one observed target (as in method A) or several targets' data is put to files common to all observed target traffic through MF (as in method B).

The maximum set of allowed characters in interception file names are "a"…"z", "A"…"Z", "-", "_", ".", and decimals "0"…"9".

File naming method A):

<LIID>_<seq>.<ext>

LIID = 	This field has a character string (or digit string for sub-address option) value, e.g. "ABCD123456". This is a unique interception request identifier allocated by the ADMF. It will be given by the ADMF to the LEA via the HI1 interface after the ADMF has been authorized to command the start of the interception of a specific target. The possible network operator identifier part used should be agreed with (and allocated by) the regulatory organization administrating the local telecommunication practises.

seq = 	integer ranging between [0..2^64-1], in ASCII form (not exceeding 20 ASCII digits), identifying the sequence number for file transfer from this node per a specific target.

ext = 	ASCII integer ranging between ["1".."7".]  (in hex: 31H…37H), identifying the file type. The possible file type coding for IRI is shown in table A.1.

Table A.1: Possible file types

| File types that the LEA may get | Intercepted data types |
| --- | --- |
| "1" (in binary: 0011 0001) | IRI |

This alternative A is used when each target's IRI is gathered per observed target to dedicated delivery files. This method provides the result of interception in a very refined form to the LEAs, but requires somewhat more resources in the MF than alternative B. With this method, the data sorting and interpretation tasks of the LEMF are considerably easier to facilitate in near real time than in alternative B.

File naming method B):

The other choice is to use monolithic fixed format file names (with no trailing file type part in the file name):

<filenamestring>   (e.g. ABXY00041014084400001)

where:

ABXY = Source node identifier part, used for all files by the mobile network operator "AB" from this MF node named "XY".

00 = year 2000

04 = month April

10 = day 10

14 = hour

08 = minutes

44 = seconds

0000 = extension

1 = file type.  The type "1" is reserved for IRI data files. (Codings "2" = CC(MO), "4" = CC(MT), "6" = CC(MO&MT) are reserved for HI3).

This alternative B is used when several targets' intercepted data is gathered to common delivery files. This method does not provide the result of interception in as refined form to the LEAs as the alternative A, but it is faster in performance for the MF point of view. With this method, the MF does not need to keep many files open like in alternative A.

## A.2.3	Profiles (this chapter is informative only)

As there are several ways (usage profiles) how data transfer can be arranged by using the FTP, this chapter contains practical considerations how the communications can be set up.  Guidance is given for client-server arrangements, session establishments, time outs, the handling of the files (in RAM or disk). Example batch file is described for the case that the sending FTP client uses files. If instead (logical) files are sent directly from the client’s RAM memory, then the procedure can be in principle similar though no script file would then be needed.

At the LEMF side, FTP server process is run, and at MF, FTP client. No FTP server (which could be accessed from outside the operator network) shall run in the MF. The FTP client can be implemented in many ways, and here the FTP usage is presented with an example only. The FTP client can be implemented by a batch file or a file sender program that uses FTP via an API. The login needs to occur only once per e.g. <destaddr> & <leauser> -pair. Once the login is done, the files can then be transferred just by repeating 'mput' command and checking the transfer status (e.g. from the API routine return value). To prevent inactivity timer triggering, a dummy command (e.g. 'pwd') can be sent every T seconds (T should be less than L, the actual idle time limit). If the number of FTP connections is wanted to be as minimised as possible, the FTP file transfer method "B" is to be preferred to the method A (though the method A helps more the LEMF by pre-sorting the data sent).

Simple example of a batch file extract:

FTP commands usage scenario for transfering a list of files:

To prevent FTP cmd line buffer overflow the best way is to use wildcarded file names, and let the FTP implementation do the file name expansion (instead of shell). The number of files for one mput is not limited this way:

ftp <flags> <destaddr>

user <leauser> <leapasswd>

cd <destpath>

lcd <srcpath>

bin

mput <files>

nlist <lastfile> <checkfile>

close

EOF

This set of commands opens an FTP connection to a LEA site, logs in with a given account (auto-login is disabled), transfers a list of files in binary mode, and checks the transfer status in a simplified way.

Brief descriptions for the FTP commands used in the example:

user <user-name> <password>	Identify the client to the remote FTP server.

cd <remote-directory>          	Change the working directory on the remote machine to remote-directory.

lcd <directory>                	Change the working directory on the local machine.

bin                            	Set the file transfer type to support binary image transfer

mput <local-files>   	Expand wild cards in the list of local files given as arguments and do a put for each file in the resulting list. Store each local file on the remote machine.

nlist <remote-directory> <local-file>	Print a list of the files in a directory on the remote machine. Send the output to local-file.

close                           	Terminate the FTP session with the remote server, and return to the command interpreter.  Any defined macros are erased.

The parameters are as follows:

<flags> 	contains the FTP command options, e.g. "-i -n -V -p" which equals to 'interactive prompting off', 'auto-login disabled', 'verbose mode disabled', and 'passive mode enabled'. (These are dependent on the used ftp- version.)

<destaddr>	contains the IP address or DNS address of the destination (LEA).

<leauser>	contains the receiving (LEA) username.

<leapasswd>	contains the receiving (LEA) user's password.

<destpath>	contains the destination path.

<srcpath>	contains the source path.

<files>	wildcarded file specification (matching the files to be transferred)

<lastfile>	the name of the last file to be transferred

<checkfile>	is a (local) file to be checked upon transfer completion; if it exists then the transfer is considered successful.

The FTP application should to do the following things if the checkfile is not found:

-	keep the failed files.

-	raise 'file transfer failure' error condition (i.e. send alarm to the corresponding LEA).

-	the data can be buffered for a time that the buffer size allows. If that would finally be exhausted, DF would start dropping the corresponding target's data until the transfer failure is fixed.

-	the transmission of the failed files is retried until the transfer eventually succeeds. Then the DF would again start collecting the data.

-	upon successful file transfer the sent files are deleted from the DF.

The FTP server at LEMF shall not allow anonymous login of an FTP client.

## A.2.4	File content

The file content is in method A relating to only one intercepted target.

In the file transfer method B, the file content may relate to any intercepted targets whose intercept records are sent to the particular LEMF address.

Individual IRI records shall not be fragmented into separate files at the FTP layer.

## A.2.5	Exceptional procedures

Overflow at the receiving end (LEMF) is avoided due to the nature of the protocol.

In case the transit network or receiving end system (LEMF) is down for a reasonably short time period, the local buffering at the MF will be sufficient as a delivery reliability backup procedure.

In case the transit network or receiving end system (LEMF) is down for a very long period, the local buffering at the MF may have to be terminated. Then the following intercepted data coming from the intercepting nodes to the MF would be discarded, until the transit network or LEMF is up and running again.

## A.2.6	Other Considerations

The FTP protocol mode parameters used:

Transmission Mode:	stream

Format:			non-print

Structure:			file-structure

Type:			binary

The FTP client (=user -FTP process at the MF) uses e.g. the default standard FTP ports 20 (for data connection) and 21 (for control connection), 'passive' mode is supported. The data transfer process listens the data port for a connection from a server-FTP process.

For the file transfer from the MF to the LEMF(s) e.g. the following data transfer parameters are provided for the FTP client (at the MF):

-	transfer destination (IP) address, e.g. "194.89.205.4"

-	transfer destination username, e.g. "LEA1"

-	transfer destination directory path, e.g. "/usr/local/LEA1/1234-8291"

-	transfer destination password

-	interception file type, "1" (this is needed only if the file naming method A is used)

LEMF may use various kind directory structures for the reception of interception files. It is strongly recommended that at the LEMF machine the structure and  access and modification rights of the storage directories are adjusted to prevent unwanted directory operations by a FTP client.

Timing considerations for the HI2 FTP transmission

The MF and LEMF sides control the timers to ensure reliable, near-real time data transfer. The transmission related timers are defined within the lower layers of the used protocol and are out of scope of this document.

The following timers may be used within the LI application:

Table A.2: Timing considerations

| Name | Controlled by | Units | Description |
| --- | --- | --- | --- |
| T1 inactivity timer | LEMF | Seconds | Triggered by no activity within the FTP session (no new files). The FTP session is torn down when the T1 expires. To send another file the new connection will be established. The timer avoids the FTP session overflow at the LEMF side. |
| T2 send file trigger | MF | Milliseconds | Forces the file to be transmitted to the LEMF (even if the size limit has not been reached yet in case of volume trigger active). If the timer is set to 0 the only trigger to send the file is the file size parameter (Ref. C.2.2). |

Annex B (normative):
Structure of data at the handover interface

- This annex specifies the coding details at the handover interface HI for all data, which may be sent from the NWO/AP/SvP's equipment to the LEMF, across HI.

- At the HI2 and HI3 handover interface ports, the following data may be present:

-	interface port HI2: Intercept related information (IRI);

-	interface port HI3: records containing content of communication (CC).

The detailed coding specification for these types of information is contained in this annex, including sufficient details for a consistent implementation in the NWO/AP/SvP's equipment and the LEMF.

It must be noticed some data are ROSE specific and have no meaning when FTP is used. Those specificities are described at the beginning of each sub-annex.

# B.1	Syntax definitions

The transferred information and messages are encoded to be binary compatible with [33] (Abstract Syntax Notation One (ASN.1)) and [34] (Basic Encoding Rules (BER)).

These recommendations use precise definitions of the words type, class, value, and parameter. Those definitions are paraphrased below for clarity.

A type, in the context of the abstract syntax or transfer syntax, is a set of all possible values. For example, an INTEGER is a type for all negative and positive integers.

A class, in the context of the abstract syntax or transfer syntax, is a one of four possible domains for uniquely defining a type. The classes defined by ASN.1 and BER are: UNIVERSAL, APPLICATION, CONTEXT, and PRIVATE.

The UNIVERSAL class is reserved for international standards such as [33] and [34]. Most parameter type identifiers in the HI ROSE operations are encoded as CONTEXT specific class. Users of the protocol may extend the syntax with PRIVATE class parameters without conflict with the present document, but risk conflict with other users' extensions. APPLICATION class parameters are reserved for future extensions.

A value is a particular instance of a type. For example, five (5) is a possible value of the type INTEGER.

A parameter in the present document is a particular instance of the transfer syntax to transport a value consisting of a tag to identify the parameter type, a length to specify the number of octets in the value, and the value.

In the BER a tag (a particular type and class identifier) may either be a primitive or a constructor. A primitive is a pre-defined type (of class UNIVERSAL) and a constructor consists of other types (primitives or other constructors). A constructor type may either be IMPLICIT or EXPLICIT. An IMPLICIT type is encoded with the constructor identifier alone. Both ends of a communication must understand the underlying structure of the IMPLICIT types. EXPLICIT types are encoded with the identifiers of all the contained types. For example, an IMPLICIT Number of type INTEGER would be tagged only with the Number tag, where an EXPLICIT number of type INTEGER would have the INTEGER tag within the Number tag. The present document uses IMPLICIT tagging for more compact message encoding.

For the coding of the value part of each parameter the general rule is to use a widely use a standardized format when it exists (ISUP, DSS1, MAP, …).

As a large part of the information exchanged between the user's may be transmitted within ISUP/DSS1 signalling, the using of the coding defined for this signalling guarantee the integrity of the information provided to the LEMF and the evolution of the interface. For example if new values are used within existing ISUP parameters, this new values shall be transmitted transparently toward the LEMF.

# B.2	Object tree

- ASN.1 description of security object tree

.$SecurityDomainDefinitions { ccitt (0) identified-organization (4) etsi (0) securityDomain (2)}

- DEFINITIONS IMPLICIT TAGS ::=

- BEGIN

-- Security DomainId

securityDomainId OBJECT IDENTIFIER	::= { ccitt (0) identified-organization (4) etsi (0) securityDomain (2)}

-- Security Subdomains

fraudSubDomainId OBJECT IDENTIFIER				::= {securityDomainId fraud (1)}

lawfulInterceptSubDomainId OBJECT IDENTIFIER	::= {securityDomainId lawfulIntercept (2)}

-- LawfulIntercept Subdomains

hi2DomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId hi2 (1)}

hi3DomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId hi3 (2)}

himDomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId him (3)}

-- HI3 Subdomains

hi3GPRSLISubDomainId OBJECT IDENTIFIER	::= {hi3DomainId gPRSLI (3)}

-- For further study

hi3GSMLISubDomainId OBJECT IDENTIFIER ::= {hi3DomainId gSMLI (5)}

-- For further study

END -- SecurityDomainDefinitions

# B.3	HI management operation

This data description applies only for ROSE delivery mechanism.

- ASN.1 description of HI management operation (any HI interface)

HIManagementOperations { ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) him (3) version2 (2)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

EXPORTS	sending-of-Password,

data-Link-Test,

end-Of-Connection ;

IMPORTS	OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t (2) remote-operations(4) informationObjects(5) version1(0)}

himDomainId

FROM SecurityDomainDefinitions

{ ccitt (0) identified-organization (4) etsi (0) securityDomain (2)};

sending-of-Password	OPERATION ::=

{

ARGUMENT	Password-Name

ERRORS 		{ ErrorsHim }

CODE		global:{ himDomainId sending-of-Password (1) version1 (1)}

}

-- Class 2 operation . The timer must be set to a value between 3 s and 240s.

-- The timer default value is 60s.

data-Link-Test		OPERATION ::=

{

ERRORS 		{ other-failure-causes }

CODE		global:{ himDomainId data-link-test (2) version1 (1)}

}

-- Class 2 operation . The timer must be set to a value between 3s and 240s.

--The timer default value is 60s.

end-Of-Connection	OPERATION ::=

{

ERRORS 		{ other-failure-causes }

CODE		global:{ himDomainId end-of-connection (3) version1 (1)}

}

-- Class 2 operation . The timer must be set to a value between 3s and 240s.

-- The timer default value is 60s.

other-failure-causes 	ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter		ERROR ::= { CODE local:2}

erroneous-parameter		ERROR ::= { CODE local:3}

ErrorsHim ERROR ::=

{

other-failure-causes |

missing-parameter |

unknown-parameter |

erroneous-parameter }

Password-Name			::= SEQUENCE {

password	[1] OCTET STRING (SIZE (1..25)),

name		[2] OCTET STRING (SIZE (1..25)),

...}

--  IA5 string recommended

END -- HIManagementOperations

# B.4	Intercept related information (HI2)

Declaration of ROSE operation sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data IRI-content must be considered.

- ASN1 description of IRI (HI2 interface)

HI2Operations { ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) hi2 (1)  version2 (2)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

EXPORTS	sending-of-IRI,

CommunicationIdentifier,

TimeStamp,

OperationErrors,

SMS-report,

LawfulInterceptionIdentifier,

Supplementary-Services,

CC-Link-Identifier;

IMPORTS OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t (2) remote-operations(4) informationObjects(5) version1(0)}

hi2DomainId

FROM

SecurityDomainDefinitions

{ ccitt (0) identified-organization (4) etsi (0) securityDomain (2)};

sending-of-IRI  OPERATION ::=

{

ARGUMENT	IRIContent

ERRORS		{ OperationErrors }

CODE		global:{ hi2DomainId sending-of-IRI (1)  version1 (1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

IRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Begin-Record

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Continue-Record

iRI-Report-record		[4] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Report-Record

...

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

--This values may be sent by the LEMF, when an operation or a parameter is misunderstood,

IRI-Parameters		::= SEQUENCE

{

iRIversion						[23] ENUMERATED

{

version2(2),

…

} OPTIONAL,

-- if not present, it means version 1 is handled

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier ,

--This identifier is associated to the target.

communicationIdentifier					[2] CommunicationIdentifier,

--used to uniquely identify an intercepted call.

-- called CallIdentifier in Edition 1 of the document

timeStamp						[3] TimeStamp,

--date and time of the event triggering the report.)

intercepted-Call-Direct 		[4] ENUMERATED

{

not-Available(0),

originating-Target(1),

-- in case of GPRS, this indicates that the PDP context activation

-- or deactivation is MS requested

terminating-Target(2),

-- in case of GPRS, this indicates that the PDP context activation or deactivation is

-- network initiated

...

} OPTIONAL,

locationOfTheTarget				[8] Location OPTIONAL,

--location of the target subscriber

partyInformation 				[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

--This parameter provides the concerned party (Originating, Terminating or forwarded party),

-- the identiy(ies) of the party and all the information provided by the party.

nature-Of-The-intercepted-call 	[12] ENUMERATED

{

--Nature of the intercepted „call„ :

gSM-SMS-Message(1),	--the SMS content is sent through the HI2 or HI3  „data„ interface

gPRS-Packet-Data(5),

--the data are sent through the HI3 „data„ interface

...

} OPTIONAL,

serverCenterAddress		[13] PartyInformation OPTIONAL,

--e.g. in case of SMS message this parameter provides the address of  the relevant

--server within the calling (if server is originating) or called (if server is terminating)

-- party address parameters

sMS						[14] SMS-report OPTIONAL,

--this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

gPRSCorrelationNumber	[18] GPRSCorrelationNumber OPTIONAL,

gPRSevent 				[20] GPRSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

...

}

-- PARAMETERS FORMATS

CommunicationIdentifier		::= SEQUENCE

{

communication-Identity-Number	[0] OCTET STRING (SIZE (1 .. 8)) OPTIONAL,

--Temporary Identifier of an intercepted call to uniquely identify an intercepted call

--within the node (free format). This parameter is mandatory if there is associated

--information sent over HI3interface (CClink, data,..) or when

--CommunicationIdentifier is used for IRI other than IRI-Report-recor

--This parameter was called call-Identity-Number in Ed.1 of the document

network-Identifier		[1] Network-Identifier,

...

}

--NB : The same „CommunicationIdentifier„ value is sent :

--with the HI3 information  for correlation purpose between  the IRI and the

--information sent on the HI3 interfaces (CCLink, data, ..)

--with each IRI associated to a same intercepted call for correlation purpose between

--the different IRI

Network-Identifier		::= SEQUENCE

{

operator-Identifier			[0] OCTET STRING (SIZE (1 .. 5)),

--it's a notification of the NWO/AP/SvP in ASCII- characters

--the parameter is mandatory.

network-Element-Identifier	[1] Network-Element-Identifier OPTIONAL,

...

}

Network-Element-Identifier	::= CHOICE

{

iP-Format			[3] OCTET STRING    (SIZE (1 .. 25)),

--IP address

dNS-Format			[4] OCTET STRING    (SIZE (1 .. 25)),

--DNS address

...

}

TimeStamp 				::= CHOICE

{

localTime			[0] LocalTimeStamp,

utcTime				[1] UTCTime

}

--The UTC Time is an ASN1 universal class and its format is the one defined

--in case b) of  the ASN1 recommendation [33] (year month day

--hour minutes seconds)

LocalTimeStamp				::= SEQUENCE

{

generalizedTime					[0] GeneralizedTime,

--The generalized Time format is an ASN1 universal class and its format is the

--one defined in case a) of the ASN1 recommendation [33], b) (year

--month day hour minutes seconds)

winterSummerIndication			[1] ENUMERATED

{

notProvided(0),

winterTime(1),

summerTime(2),

...

}

}

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

.

gPRS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei				[1] OCTET STRING (SIZE (8)) OPTIONAL,

--See MAP format [32]

imsi				[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

--See MAP format [32] International Mobile

--Station Identity E.212 number beginning with Mobile Country Code

msISDN				[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document ref [32], § 14.7.8

...

},

services-Data-Information [4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the complementary

-- information associated to the basic data call

...

}

Location	::= SEQUENCE

{

globalCellID		[2] OCTET STRING  (SIZE (5..7)) OPTIONAL,

--see MAP format (see [32])

,

rAI					[4] OCTET STRING (SIZE (6)) OPTIONAL,

-- the Routeing Area Identifier is coded in accordance with the § 10.5.5.15 of

-- document ref [41] without the Routing Area Identification IEI (only the

-- last 6 octets are used)

gsmLocation			[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] OCTET STRING  (SIZE (7)) OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 – 3),

--			LAC		2 octets (no. 4 – 5),

--			SAC		2 octets (no. 6 – 7)

--			(according to 3GPP TS 25.413)

...

}

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude	[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude	[2]	PrintableString (SIZE(8..11))

-- format : 	XDDDMMSS.SS

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 	[1] PrintableString (SIZE(10)),

utm-North 	[2] PrintableString (SIZE(7))

-- example	utm-East	32U0439955

--		utm-North	   5540736

},

utmRefCoordinates  	[3] PrintableString (SIZE(13)),

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING (SIZE(7..10))

-- format is as defined in GSM 03.32; polygon type of shape is not allowed.

}

- UMTSLocation ::= CHOICE {

point					[1]		GA-Point,

pointWithUnCertainty	[2]		GA-PointWithUnCertainty,

polygon					[3]		GA-Polygon,

...

}

- GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

- GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

- GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

- GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

- SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

SMS-report		::= SEQUENCE

{

communicationIdentifier				[1] CommunicationIdentifier,

-- used to uniquely identify an intercepted call : the same used for the

-- relevant IRI

-- called CallIdentifier in Ed.1 of the document

timeStamp				[2] TimeStamp,

--date and time of the report. The format is

--the one defined in case a) of the ASN1 recommendation [33].

--(year month day hour minutes seconds)

sMS-Contents			[3] SEQUENCE

{

initiator				[1] ENUMERATED

{

--party which sent the  SMS

target(0),

server(1),

undefined-party(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer(0), --the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED

{

--in case of terminating call, indicates if the server will send

--other SMS

yes(0),

no(1),

undefined(2),

...

} OPTIONAL,

content			[4] OCTET STRING (SIZE (1 .. 270)) ,

--Encoded in the format defined for the SMS mobile

...

}

}

LawfulInterceptionIdentifier	::= OCTET STRING (SIZE (1..25))

It is recommended to use ASCII characters in "a"…"z", "A"…"Z", "-", "_", ".", and "0"…"9"

--For sub-address option only "0"..."9" shall be us

National-Parameters		::= SET SIZE (1..40) OF OCTET STRING (SIZE (1..256))

--Content defined by national law

GPRSCorrelationNumber ::= OCTET STRING (SIZE(8))

GPRSEvent ::= ENUMERATED

{

pDPContextActivation(1),

startOfInterceptionWithPDPContextActive(2),

pDPContextActivationUnsuccessful (3)

pDPContextDeactivation(4),

gPRSAttach (5),

gPRSDetach (6),

gPRSAttachUnsuccessful (7)

endOfInterceptionWithPDPContextActive(8)

pDPContextModification (9)

cellOrRAUpdate (10),

sMS (11),

...

}

-- see ref [42]

Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

...

}

GPRSOperationErrorCode ::= OCTET STRING (SIZE(2))

-- refer to standard [41] for values(GMM cause or SM cause parameter).

DataNodeAddress ::= CHOICE

{

ipAddress 	[1] IPAddress,

x25Address 	[2] X25Address,

...

}

IPAddress ::= SEQUENCE

{

iP-type [1] ENUMERATED

{

iPV4(0),

iPV6(1),

...

},

iP-value [2] IP-value,

...

}

IP-value ::= CHOICE

{

iPBinaryAddress	[1] OCTET STRING (SIZE(4..16)),

iPTextAddress	[2] IA5String (SIZE(7..45)),

...

}

X25Address ::= OCTET STRING (SIZE(1..25))

END -- OF HI2Operations

Annex C (normative):
GPRS HI3 Interface

There are two possible methods for delivery of content of communication to the LEMF:

-	GPRS LI Correlation Header and UDP/TCP

-	FTP

According to national requirements al least one of these methods have to be provided.

# C.1	GPRS LI Correlation Header

## C.1.1	Introduction

The header and the payload of the communication between the intercepted subscriber and the other party (later called: Information Element) is duplicated. A new header (later called: GLIC-Header, see figure C.1) is added (see figure C.3) before it is sent to LEMF.

Data packets with the GLIC header shall be sent to the LEA  via UDP or TCP/IP.

## C.1.2	Definition of GLIC Header

GLIC header contains the following attributes:

- Correlation Number

- Message Type (a value of 255 is used for HI3-PDU’s).

- Direction

- Sequence Number

- Length

T-PDU contains the intercepted information

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| 1 |  | Version (‘0 0 0’) |  |  | ‘1’ | Spare ‘1 1‘ |  | DIR |  | ‘0’ |
| 2 |  | Message Type (value 255) |  |  |  |  |  |  |  |  |
| 3-4 |  | Length |  |  |  |  |  |  |  |  |
| 5-6 |  | Sequence Number |  |  |  |  |  |  |  |  |
| 7-8 |  | not used (value 0) |  |  |  |  |  |  |  |  |
| 9 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 10 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 11 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 12 |  | not used (value 255) |  |  |  |  |  |  |  |  |
| 13-20 |  | correlation number |  |  |  |  |  |  |  |  |

- Figure C.1: Outline of GLIC header

-	For interception tunneling  the GLIC header shall be used as follows:

-	Version shall be set to 0 to indicate the first version of  GLIC header.

-	DIR indicates the direction of the T-PDU:

"1" indicating uplink (from observed mobile user) and

"0" indicating downlink (to observed mobile user).

-	Message Type shall be set to 255 (the unique value that is used for T-PDU within GTP [45]).

-	Length shall be the length, in octets, of the signaling message excluding the GLIC header. Bit 8 of octet 3 is the most significant bit and bit 1 of octet 4 is the least significant bit of the length field.

-	Sequence Number is an increasing sequence number for tunneled T-PDUs. Bit 8 of octet 5 is the most significant bit and bit 1 of octet 6 is the least significant bit of the sequence number field.

-	Correlation Number consists of two parts:-	GGSN-ID identifies the GGSN which creates the Charging-ID

Charging-ID is defined in [45] and assigned uniquely to each PDP context activation on that GGSN (4 octets).

The correlation number consist of 8 octets and guarantees a unique identification of the tunnel to the LEA over a long time. The requirements for this identification are similar to that defined for charging in [45], chapter 5.4. Therefore it is proposed to use the Charging-ID,  defined in [45] , chapter 5.4 as part of correlation number. The Charging-ID is signaled to the new SGSN in case of SGSN-change so the tunnel identifier could be used “seamlessly” for the HI3 interface.

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |  |
| Charging –ID Octet 1 |  |  |  |  |  |  |  | Charging –ID Octet 2 |  |  |  |  |  |  |  | Charging –ID Octet 3 |  |  |  |  |  |  |  | Charging –ID Octet 4 |  |  |  |  |  |  |  | Octet 13-16 |
| GGSN-ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 17-20 |

Figure C.2: Outline of correlation number

The GLIC header is followed by a subsequent payload information element. Only one information element is allowed in a single signalling message.

|  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 – 20 | GLIC -Header |  |  |  |  |  |  |  |
| 21 –n | Information Element |  |  |  |  |  |  |  |

- Figure C.3: GLIC header followed by the subsequent payload Information Element

The Information Element contains the header and the payload of the communication between the intercepted subscriber and the other party.

## C.1.3	Exceptional Procedure

With UDP and GLIC: the delivering node doesn’t take care about any problems at LEMF.

With TCP and GLIC: TCP tries to establish a connection to LEMF and resending (buffering in the sending node) of packets is also supported by TCP.

In both cases it might happen that call content gets lost (in case the LEMF or the transit network between MF and LEMF is down for a long time).

## C.1.4	Other Considerations

The use of IPsec for this interface is recommended.

The required functions in LEMF are:

-	Collecting and storing of the incoming packets inline with the sequence numbers.

-	Correlating of CC to IRI with the use of the correlation number in the GLIC header.

# C.2	FTP

## C.2.1	Introduction

At HI3 interface FTP is used over the internet protocol stack for the delivery of the result of interception. FTP is defined in ref [46]. The IP is defined in ref [51]. The TCP is defined in ref [52].

FTP supports reliable delivery of data. The data may be temporarily buffered in the sending node (MF) in case of link failure. FTP is independent of the payload data it carries.

## C.2.2	Usage of the FTP

In the packet data LI the MF acts as the FTP client and the receiving node (LEMF) acts as the FTP server . The client pushes the data to the server.

The receiving node LEMF stores the received data as files. The sending entity (MF) may buffer files.

Several smaller intercepted data units may be gathered to bigger packages prior to sending, to increase bandwidth efficiency.

The following configurable intercept dta collection (= transfer package closing / file change) threshold parameters should be supported:

-	frequency of transfer, based on send timeout, e.g. X ms

-	frequency of transfer, based on volume trigger, e.g. X octets

There are two possible ways how the interception data may be sent from the MF to the LEMF. One way is to produce files that contain interception data only for one observed target (ref: "File naming method A)"). The other way is to multiplex all the intercepted data that MF receives to the same sequence of general purpose interception files sent by the MF (ref: "File naming method B)" ).

The HI2 and HI3 are logically different interfaces, even though in some installations the HI2 and HI3 packet streams might also be delivered via a common transmission path from a MF to a LEMF. It is possible to correlate HI2 and HI3 packet streams by having common (referencing) data fields embedded in the IRI and the CC packet streams.

- File naming:

The names for the files transferred to a LEA are formed according to one of the 2 available formats, depending on the delivery file strategy chosen (e.g. due to national convention or operator preference).

Either each file contains data of only one observed target (as in method A) or several targets' data is put to files common to all observed target traffic through a particular MF node (as in method B).

The maximum set of allowed characters in interception file names are "a"…"z", "A"…"Z", "-", "_", ".", and decimals "0"…"9".

- File naming method A):

<LIID>_<seq>.<ext>

LIID = This field has a character string value, e.g. "ABCD123456". This is a unique interception request identifier allocated by the ADMF. It will be given by the ADMF to the LEA via the HI1 interface after the ADMF has been authorised to command the start of the interception of a specific target. The possible  network operator identifier part used should be agreed with (and allocated by) the regulatory organization administrating the local telecommunication practises.

Seq = integer ranging between [0..2^64-1], in ASCII form (not exceeding 20 ASCII digits), identifying the sequence number for file transfer from this node per a specific target.

Ext = ASCII integer ranging between ["1".."7".]  (in hex: 31H…37H), identifying the file type. The possible file type codings for intercepted data are shown in table C.1. But for the HI3 interface, only the types "2", "4", and "6" are possible.

- Table C.1: Possible file types

| File types that the LEA may get | Intercepted data types |
| --- | --- |
| "2" (in binary: 0011 0010) | CC(MO) |
| "4" (in binary: 0011 0100) | CC(MT) |
| "6" (in binary: 0011 0110) | CC(MO&MT) |

(The least significant bit that is ‘1’ in file type 1, is reserved for indicating IRI data.) The bit 2 of the ext tells whether the Mobile Originated (MO) Content of Communication (CC) is included to the intercepted data.

The bit 2 of the ext tells whether the Mobile Originated (MO) Content of Communication (CC) is included to the intercepted data.

The bit 3 of the ext tells whether the Mobile Terminated (MT) Content of Communication (CC) is included to the intercepted data.

Thus, for Mobile Originated Content of Communication data, the file type is "2", for MT CC data "4" and for MO&MT CC data "6".

This alternative A is used when each target's intercepted data is gathered per observed target to dedicated delivery files. This method provides the result of interception in a very refined form to the LEAs, but requires somewhat more resources in the sending node than alternative B. With this method, the data sorting and interpretation tasks of the LEMF are considerably easier to facilitate in near real time than in alternative B.

- File naming method B):

The other choice is to use monolithic fixed format file names (with no trailing file type part in the file name):

<filenamestring>   (e.g. ABXY00041014084400006)

where:

ABXY = Source node identifier part, used for all files by the mobile network operator "AB" from this MF node named "XY".

00 = year 2000

04 = month April

10 = day 10

14 = hour

08 = minutes

44 = seconds

0000 = extension.

6 = file type. Coding: "2" = CC(MO), "4" = CC(MT), "6" = CC(MO&MT). (The type "1" is reserved for IRI data files).

This alternative B is used when several targets' intercepted data is gathered to common delivery files. This method does not provide the result of interception in as refined form to the LEAs as the alternative A, but it is faster in performance for the MF point of view. With this method, the MF does not need to keep many files open like in alternative A.

## C.2.3	Exceptional procedures

Overflow at the receiving end (LEMF) is avoided due to the nature of the protocol.

In case the transit network or receiving end system (LEMF) is down for a reasonably short time period, the local buffering at the MF will be sufficient as a delivery reliability backup procedure.

In case the transit network or receiving end system (LEMF) is down for a very long period, the local buffering at the MF may have to be terminated. Then the following intercepted data coming from the intercepting nodes towards the MF would be discarded, until the transit network or LEMF is up and running again.

## C.2.4	CC Contents for FTP

### C.2.4.1	Fields

The logical contents of the CC-header is described here.

CC-header = (Version, HeaderLength, PayloadLength, PayloadType, PayloadTimeStamp, PayloadDirection,  CCSeqNumber, CorrelationNumber, LIID, PrivateExtension)

The Information Element CorrelationNumber forms the means to correlate the IRI and CC of the communication session intercepted.

The first column indicates whether the Information Element referred is Mandatory, Conditional or Optional.

The second column is the Type in decimal.

The third column is the length of the Value in octets.

(Notation used in table C.2:  M = Mandatory, O = Optional, C= Conditional.)

Table C.2: Information elements in the CC header

| Mode | Type | Length | Value |
| --- | --- | --- | --- |
| M | 130 | 2 | Version = the version number of the format version to be used. This field has a decimal value, this enables version changes to the format version. The values are allocated according to national conventions. |
| O | 131 | 2 | HeaderLength = Length of the CC-header up to the start of the payload. octets. (This field is optional since it is useful only in such cases that these information elements would be transferred without a dynamic length encapsulation that contains all the length information anyway. This field could be needed in case of e.g. adapting to a local encapsulation convention.) |
| O | 132 | 2 | PayloadLength = Length of the payload following the CC-header. (This field is optional since it is useful only in such cases that these information elements would be transferred without a dynamic length encapsulation that contains all the length information anyway. This field could be needed in case of e.g. adapting to a local encapsulation convention.) |
| M | 133 | 1 | PayloadType = Type of the payload, indicating the type of the CC. Type of the payload. This field has a decimal value. The possible PDP Type values can be found in the standards, e.g. GSM 09.60 [45]. The value 255 is reserved for future PDP Types and means: "Other". |
| O | 134 | 4 | PayloadTimeStamp = Payload timestamp according to intercepting node. (Precision: 1 second, timezone: UTC). Format: Seconds since 1970-01-01 as in e.g. Unix (length: 4 octets). |
| C | 137 | 1 | PayloadDirection = Direction of the payload data. This field has a decimal value 0 if the payload data is going towards the target (ie. downstream), or 1 if the payload data is being sent from the target (ie. upstream). If this information is transferred otherwise, e.g. in the protocol header, this field is not required as mandatory. If the direction information is not available otherwise, it is mandatory to include it here in the CC header. |
| O | 141 | 4 | CCSeqNumber = Identifies the sequence number of each CC packet during interception of the target. This field has a 32-bit value. |
| M | 144 | 8 or 20 | CorrelationNumber. Identifies an intercepted session of the observed target. This can be implemented by using e.g. the Charging Id (4 octets, see [49]) with the (4-octet/16-octet) Ipv4/Ipv6 address of the PDP context maintaining GGSN node attached after the first 4 octets. |
|  |  |  | <Possible future parameters are to be allocated between 145 and 253.> |
| O | 254 | 1-25 | LIID = Field indicating the LIID as defined in this document. This field has a character string value, e.g. "ABCD123456". |
| O | 255 | 1-N | PrivateExtension = An optional field. The optional Private Extension contains vendor or LEA or operator specific information. It is described in the document GSM 09.60 [45] |

### C.2.4.2	Information Element Syntax

The dynamic TypeLengthValue (TLV) format is used for ist ease of implementation and good encoding and decoding performance. Subfield sizes: Type = 2 octets, Length = 2 octets and Value = 0…N octets. From Length the T and L subfields are excluded. The Type is different for every different field standardized.

The octets in the Type and Length subfields are ordered in the little-endian order, (i.e. least significant octet first). Any multioctet Value subfield is also to be interpreted as being little-endian ordered (word/double word/long word) when it has a (hexadecimal 2/4/8-octet) numeric value, instead of being specified to have an ASCII character string value. This means that the least significant octet/word/double word is then sent before the more significant octet/word/double word.

TLV encoding:

| Type (2 octets) | Length (2 octets) | Value (0-N octets) |
| --- | --- | --- |

Figure C.4: Information elements in the CC header

- TLV encoding can always be applied in a nested fashion for structured values.

| T | L | V | T | L | V TLV TLV TLV TLV |  |
| --- | --- | --- | --- | --- | --- | --- |

(The small "v" refers to the start of a Value field that has inside it a nested structure.)

Figure C.5: Information elements in the CC header

In figure C.6, the TLV structure for GPRS HI3 transfer is presented for the case that there is just one intercepted packet inside the CC message. (There can be more CC Header IEs and CC Payload IEs in the CC, if there are more intercepted packets in the same CC message.)

Figure C.6: IE structure of a CC message that contains one intercepted packet

The first octet of the first TLV element will start right after the last octet of the header of the protocol that is being used to carry the CC information.

The first TLV element (i.e. the main TLV IE) comprises the whole dynamic length CC information, i.e. the dynamic length CC header and the dynamic length CC payload.

Inside the main TLV IE there are at least 2 TLV elements: the Header of the payload and the Payload itself. The Header contains all the ancillary IEs related to the intercepted CC packet. The Payload contains the actual intercepted packet.

There may be more than one intercepted packet in one GPRS HI3 delivery protocol message. If the Value of the main TLV IE is longer than the 2 (first) TLV Information Elements inside it, then it is an indication that there are more than one intercepted packets inside the main TLV IE (i.e. 4 or more TLV IEs in total). The number of TLV IEs in the main TLV IE is always even, since for every intercepted packet there is one TLV IE for header and one TLV IE for payload.

## C.2.5	Other Considerations

The FTP protocol mode parameters used:

Transmission Mode:	stream

Format:	non-print

Structure:	file-structure

Type:	binary

The FTP service command to define the file system function at the server side: STORE mode for data transmission.

The FTP client– (=user -FTP process at the MF) uses e.g. the default standard FTP ports 20 (for data connection) and 21 (for control connection), 'passive' mode is supported. The data transfer process listens the data port for a connection from a server-FTP process.

For the file transfer from the MF to the LEMF(s) e.g. the following data transfer parameters are provided for the FTP client (at the MF):

-	transfer destination (IP) address, e.g. "194.89.205.4"

-	transfer destination username, e.g. "LEA1"

-	transfer destination directory path, e.g. "/usr/local/LEA1/1234-8291"

-	transfer destination password

-	interception file type, e.g. "2" (this is needed only if the file naming method A is used)

LEMF may use various kind directory structures for the reception of interception files. It is strongly recommended that at the LEMF machine the structure and access and modification rights of the storage directories are adjusted to prevent unwanted directory operations by a FTP client.

The use of IPSec services for this interface is recommended.

Timing considerations for the FTP transmission

The MF and LEMF sides control the timers to ensure reliable, near-real time data transfer. The transmission related timers are defined within the lower layers of the used protocol and are out of scope of this document.

The following timers may be used within the LI application:

Table C.3: Timing considerations

| Name | Controlled by | Units | Description |
| --- | --- | --- | --- |
| T1 inactivity timer | LEMF | Seconds | Triggered by no activity within the FTP session (no new files). The FTP session is torn down when the T1 expires. To send another file the new connection will be established. The timer avoids the FTP session overflow at the LEMF side. |
| T2 send file trigger | MF | Milliseconds | Forces the file to be transmitted to the LEMF (even if the size limit has not been reached yet in case of volume trigger active). If the timer is set to 0 the only trigger to send the file is the file size parameter (Ref. C.2.2). |

Annex D (informative):
LEMF Requirements - Handling of Unrecognized Fields and Parameters

During decoding of a record at the LEA, the following exceptional situations may occur:

1)	Unrecognized parameter: The parameter layout can be recognized, but its name is not recognized:
The parameter shall be ignored, the processing of the record proceeds.

2)	The parameter content or value is not recognized or not allowed:
The parameter shall be ignored, the processing of the record proceeds.

3)	The record cannot be decoded (e.g. it seems to be corrupted):
The whole record shall be rejected when using ROSE delivery mechanism or ignored.

NOTE:	In cases 2 and 3, the LEMF may wish to raise an alarm to the NWO/AP/SvP administration centre. For case 1, no special error or alarm procedures need be started at the LEA, because the reason may be the introduction of a new version of the specification in the network, not be an error as such security aspects.

Annex E (informative):
Bibliography

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

-

-

-

-

-

-

-

-

-

-

-

-

-

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| June 2001 |  |  |  |  | Initial draft |  | V0.0.0 |
| June 2001 |  |  |  |  | Revised draft - review via correspondence (e-mail discussion) | V0.0.0 | V0.0.1 |
| June 2001 |  |  |  |  | Revised draft with structural revision marks removed - circulated for review via correspondence (e-mail discussion). | V0.0.1 | V0.0.1a |
| August 2001 |  |  |  |  | Editorial Revisions by a) SA3-LI editor, and b) 3GPP standard formatting by MCC | V0.0.1a | V0.0.2 |